Technical Note

NS300 EDS File Usage

Applicable Products: Sigma 2 (SGDH) with NS300

Yaskawa Electric America

2121 Norman Drive South

Waukegan, IL 60085

1-800-927-5292

About This Document

This document provides instructions on the application of the NS300 *.eds file.

# NS300 *.eds files:

When viewing this document with Microsoft Word, double-click the below icon to execute the file extraction application that handles *.zip files.

# NS300 *.eds file usage

Use the *.eds files according to the following priority:

- NS300_Upper30.eds

  - This file is designed for NS300 Firmware version 03000 and above.

  - If this file does not work sufficiently, use the EDS file (2) below.

- NS300_Lower2F.eds

  - This file is designed for NS300 Firmware version 02FFF and below.

  - Other than the filename, this file is the exact same as the previously released file “NS300(Rotary).eds”.

  - If this file does not work sufficiently, use the EDS file (3) below.

- NS300E_Lower2F.eds

  - This file is designed for NS300 Firmware version 02FFF and below.

  - If this file does not work sufficiently, use the EDS file (4) below.

- NS300E_Upper30.eds

  - This file is designed for NS300 Firmware version 03000 and above.

# NS300 with Linear motors

No *.eds file exists for the NS300 to be used with linear motors other than “NS300(Linear).eds”

Contact Yaskawa Technical Support for assistance.

# NS300 with Direct Drive motors

No *.eds file exists for the NS300 to be used with direct drive motors.

However, the parameters can be set locally at the amplifier.

Contact Yaskawa Technical Support for assistance.