TABLE OF CONTENTS

1.	About This Document	2

2.	Requirements	2

3.	Instructions to add Sigma-5-SD component to SigmaWin+	3

4.	Instructions to download a motor file to a Sigma-5-SD SERVOPACK	4

Appendix A:	Sigma-5-SD Component Detailed Installation	10

# About This Document

This document is intended for end users of Sigma-5-SD Series products. It describes how to add the Sigma-5-SD component to SigmaWin+ Ver. 5.75 or above, and how to download a motor file to the SERVOPACK.

# Requirements

- SigmaWin+ Ver. 5.75 or above must be installed.

- Sigma-5-SD Component installer downloaded from Yaskawa website.

- Motor File downloaded from Yaskawa website and matches part number of motor to be used.

- Contact Yaskawa if trying to connect to an EtherCAT SERVOPACK.

# Instructions to add Sigma-5-SD component to SigmaWin+

- Verify requirements on section 2 are met.

- Run the Sigma-5-SD component installer. The installer’s name is SWFVSD100_SetupE.exe.

- After successfully installing this component, it will appear next time SigmaWin+ is executed.

- Execute SigmaWin+ and verify Sigma-5-SD module can be seen as in the following:

Figure 1 - Sigma-5-sd module added to sigmawin+

- If the Sigma-5-SD module was installed correctly as in Figure 1, then SigmaWin+ is ready to be connected to any Sigma-5-SD SERVOPACK.

- If the Sigma-5-SD module cannot be seen as in the figure above, try installing the component again following instructions on Appendix A.

# Instructions to download a motor file to a Sigma-5-SD SERVOPACK

- Make sure only control power (24V) is being supplied to the converter and inverter.

- Using Yaskawa USB cable, connect the SERVOPACK to SigmaWin+ Ver. 5.75 or above.

- Once connected, select Setup(S) from the menu bar and click on “Motor parameter servopack write(W)”

- Select “OK” on the next screen to start reading information from the SERVOPACK

- In the Motor parameter servopack write – File Select window, select Ref.

- Select the motor file to be downloaded to the SERVOPACK and click on Open.

- Make sure the motor file matches the motor installed, and then click on next.

- Click on write to download the motor file to the SERVOPACK.

- Click on Yes on the warning message, and the motor file would be downloaded to the SERVOPACK.

- Click on Complete on the next screen to confirm the completion of the procedure.

- Click on OK on the CAUTION message.

- Turn off and on the control power (24V) being supplied and the download procedure is now complete.

- To verify that the motor file has been downloaded successfully, open SigmaWin+.

- SigmaWin+ would normally be connected to the Sigma-5-SD SERVOPACK at this point, if not reconnect to the drive.

- Go to Monitor(M), then Product Information(P)

- In the following popup screen, verify that motor type matches the part number of the installed motor.

- If the motor type matches, then the download procedure has been done successfully; otherwise, the motor file downloaded to the drive is for a different spindle motor than the one installed. If that is the case, then download the right motor file and follow this procedure again.

- Sigma-5-SD Component Detailed Installation

- Make sure Sigma-5-SD installer’s name is SWFVSD100_SetupE.exe.

- Run SWFVSD100_SetupE.exe.\

- Click on Next on the following installation screen:

- Accept the terms of the license agreement and click on Next.

- Click on Install and wait a few seconds for the installation to be completed.

- Click on Finish to finalize the installation

- After successfully installing this component, it will appear next time SigmaWin+ is executed.